### Purpose

To aid in reducing the risks associated with storing compressed gasses on a vessel.

### Responsibilities

- Master – Ensure this procedure is carried out as written and to make any changes needed to this procedure to ensure the safety of the crew.

- Crew – Follow the procedure below at the direction of the vessel master.

### Procedure

| Step | Compressed Gas |
| --- | --- |
| 1. | All compressed gas cylinders are labeled, stored upright and secured to a wall or other mounting surface. If stowed in compartments or lockers, ensure compartments and lockers are labeled for contents. |
| 2. | Ensure a valve protection cover is placed on the cylinder when it is not in use. |
| 3. | Inspect cylinders on a regular basis for corrosion. |
| 4. | Ensure hoses, gauges and torches are intact and flashback valves are in place on oxygen acetylene tanks. |
| 5. | Store oxidizing gases away from flammable gasses or other materials when they are not in use (e.g. Acetylene) |
| 6. | Ensure adequate ventilation is available in areas where compressed gasses are stored. |
| 7. | If storing CO2 for fire suppression systems, ensure the crew is aware of the symptoms of exposure. |