### Purpose

To ensure regular preventative maintenance is being carried out on the vessel to reduce the risk of equipment failure.

### Responsibilities

- Master – Ensure this procedure is carried out as written and to make any changes needed to this procedure to ensure the safety of the crew.

- Crew – Follow the procedure below at the direction of the vessel master.

### Procedure

| Step | Preventative Maintenance (before start of season) |
| --- | --- |
| 1. | Test bilge, ballast and fire pumps, clean the strainers and service the pumps. |
| 2. | Check steady bearings and lubrication or grease lines. |
| 3. | Have life rafts serviced annually and replace hydrostatic releases as required. |
| 4. | Check batteries in detectors, emergency lights, etc. |
| 5. | Check all ship’s batteries, service and replace corroded terminals and connectors. |
| 6. | Service the main and auxiliary engines. |
| 7. | Replace all zinc anodes in heat exchangers. |
| 8. | Service the refrigeration system and test it out. |
| 9. | Check all electronic equipment and service as necessary (grease scanner if required). |
| 10. | Test the anchor winches, run out anchors and remark anchor cable. |
| 11. | Test and service domestic plumbing system and fresh water purification system. |
| 12. | Service hydraulic system, change filters if required, lubricate exposed control valves and repaint. |
| 13. | Check and service the steering system. |
| 14. | Do other maintenance checks as per other fixtures and equipment on board your particular vessel. |

* Checklist items taken from Chapter 14 of the Fishing Master Program – Book 1